1. IDENTIFICATION OF THE SUBSTANCE / PREPARATION AND THE COMPANY

PICLORAM 240G/L SL

SUPPLIER NAME:             Shandong Weifang Rainbow Chemical Co., Ltd

SUPPLIER ADDRESS:         Binhai Economic Development Area, Weifang, Shandong, China

SUPPLIER PHONE:           +86-531-88875230, 88875231

SUPPLIER FAX:              +86-531-88875232, 88875224

CHEMICAL NAME            4-amino-3,5,6-trichloropyridine-2-carboxylic acid

CAS NUMBER               1918-02-1

MOLECULAR FORMULA      C6H3Cl3N2O2

MOLECULAR WEIGHT        241.46

USE:                        Herbicide

2. COMPOSITION / INFORMATION ON INGREDIENTS

PICLORAM ACID (PRESENTED AS POTASSIUM SALT):           	240 g/l

OTHERS:      			                                       Up to 1.0 L

3. HAZARDS IDENTIFICATION

EYE CONTACT:       May cause moderate eye irritation.  Corneal injury is unlikely.

SKIN CONTACT:      Brief contact is essentially nonirritating to skin.

SKIN ABSORPTION:   Prolonged skin contact is unlikely to result in absorption of harmful amounts.

SKIN SENSITIZATION: Has caused allergic skin reactions when tested in guinea pigs.

INHALATION:         No adverse effects are anticipated from single exposure to mist.  Based on the available data, narcotic effects were not observed.  Based on the available data, respiratory irritation was not observed.

4. FIRST AID

EYES: 	Flush eyes with plenty of water.

SKIN: 	 Wash off in flowing water or shower.

INGESTION: 	If swallowed, seek medical attention. Do not induce vomiting unless directed to do so by medical personnel.

INHALATION: 	Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, oxygen should be administered by qualified personnel. Call a physician or transport to a medical facility.

NOTE TO PHYSICIAN: 	No specific antidote. Supportive care. Treatment based on judgment of the physician in response to reactions of the patient.

5. FIRE FIGHTING MEASURES

FLAMMABLE PROPERTIES: 	Not application.

FLASH POINT: 			No flammability

FLAMMABLE LIMITS: 		LFL: Information not available

UFL: Information not available

EXTINGUISHING MEDIA: 	Use fire-fighting measures, which suit the environment and take into account other materials, which may be involved. In general, water-based extinguishers should not be used for fires involving organic materials. Use carbon dioxide or dry powder.

PROTECTIVE EQUIPMENT:	Wear self-contained breathing apparatus and protective clothing.

HAZARDOUS PRODUCTS OF COMBUSTION MAY INCLUDE: Carbon monoxide, carbon dioxide, hydrogen chloride (hydrochloric acid), oxides of nitrogen, nitric acid, hydrogen cyanide.

6. ACCIDENTAL RELEASE MEASURES

PERSONAL PROTECTION: 		Wear protective equipment including rubber gloves, and eye protection. Keep unprotected persons away.

ENVIRONMENTAL PROTECTION: 	Take precautions to ensure product does not contaminate the ground or enter the drainage system.

COLLECTION: 				Mix with vermiculite or proprietary absorbent material and transfer to sealed containers for disposal.

7. HANDLING AND STORAGE

HANDLING: 			Do not get product in eyes or on clothing. Avoid contact with skin.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: Store in tightly sealed containers in a cool place. Protect from moisture.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

These precautions are suggested for conditions with a high potential for exposure. If handling procedures are such that there is only a low potential for exposure, less protection may be needed. Emergency conditions may require additional precaution.

ENGINEERING CONTROLS: 		Provide general and / or local exhaust ventilation to control airborne levels below the exposure guidelines.

EYE / FACE PROTECTION: 		Use chemical goggles. Eye wash fountain should be located in immediate work area.

SKIN: 					Use gloves impervious to this material when prolonged or frequently repeated contact could occur.

RESPIRATORY PROTECTION: 	Avoid inhalation of product. Handle in an efficient fume hood or equivalent system.

HAND AND BODY PROTECTION: 	Irritant product. Avoid skin contact. Wear rubber gloves, protective clothing and, for larger quantities, full arms, body and face protection. Wash hands thoroughly after handing.

9. PHYSICAL AND CHEMICAL PROPERTIES

APPEARANCE: 				     Liquid

COLOUR: 					     Yellow to brown

BOILING POINT (Deg C): 		     100 °C (approx.)

FLASH POINT: 				     > 100 °C (TCC)

VAPOUR PRESSURE (mm Hg): 	     22mm Hg/20 °C (approx.)

DENSITY (g/ml): 				     1.14 g/cm3 (approx.)

SOLUBILITY IN WATER: 		     Miscible

pH: 						     8.5 -11.0..

10. STABILITY AND REACTIVITY

CHEMICAL STABILITY: 				Product should be stable at normal handing and storage.

CONDITIONS TO AVOID: 				Store under cool, dry conditions. Avoid elevated temperatures and direct sunlight.

HAZARDOUS DECOMPOSITION PRODUCTS: 	Hydrogen chloride, nitrogen oxides under fire conditions, chlorinated pyridine.

HAZARDOUS POLYMERIZATION: 			Will not occur.

11. TOXICOLOGICAL INFORMATION

INHALATION:			No adverse effects anticipated by this route of exposure incidental to proper handling.

SKIN CONTACT:	The dermal LD50 for rabbits is >2000mg/kg. A single prolonged exposure is not likely to result in the material being absorbed through skin in harmful amounts.

SENSITISATION:			Sensitising to guinea pig skin.

EYE CONTACT:			May cause eye irritation.

INGESTION:			No adverse effects are anticipated by this route of exposure incidental to proper handling

12. ECOLOGICAL INFORMATION

Assessment largely or completely based on data for active ingredient.

PERSISTENCE AND DEGRADABILITY: The major route of degradation is by photolysis and photodegradation. Microbial degradation leads to formation of parent acid which undergoes further degradation. Half-life of the parent acid in soil is dependent on conditions and is approximately 10 - 278 days.

AQUATIC TOXICITY:			    Based on the acid.

Acute LC50 for daphnids is reported to be 68 mg/l

Algal toxicity EC50for green algae is 37 mg/l

Based on data for the formulation.

Dietary LC50 for sheepshead minnow (Cyprinodon variegatus) is > 100 mg/l.

AVIAN TOXICITY:				    Based on data for the formulation.

Dietary LC50 for mallard duck is > 10 mg/kg of diet

Dietary LC50 for bobwhite quail is > 10 mg/kg of diet

OTHER INFORMATION: 		    Active ingredient: Material is practically non-toxic to bees on an acute basis.

13. DISPOSAL CONSIDERATIONS

DISPOSAL METHOD: 	Do not contaminate food, feed, or water by storage or disposal. Pesticide wastes are toxic. If wastes resulting from the use of this product cannot be disposed of according to label instructions, dispose of these wastes at an approved facility.

14. TRANSPORT INFORMATION

UN number: 3082

Class: 9

Packing group: III

Marine pollutant: YES

15. REGULATORY INFORMATION

The information herein is presented in good faith and believed to be accurate as of the effective date shown above. However, no warranty, express or implied, is given.

Regulatory requirements are subject to change and may differ from one location to another; it is the buyer’s responsibility to ensure that its activities comply with federal, state or provincial, and /or local laws。

# 16. OTHER INFORMATION

The data given here is based on current knowledge and experience. The purpose of this safety data sheet is to describe the products in terms of their safety requirements. The data does not signify any warranty with regard to the products' properties.